Documentation

Fields:

a) Instructions:

decode_instr:

- sqN

- pc

- valid

- f_unit[0:1]

- oper

- rs1

- rs2

- rd

- is_imm

- imm

- br_type (0 for non-branch instructions)

- is_jump

rename_instr:

- sqN

- pc

- valid

- f_unit[0:1]

- oper

- rs1_tag

- rs2_tag

- rd_tag

- is_imm

- imm

- br_type

- is_jump

dispatch_instr:

- sqN

- pc

- valid

- oper

- rs1_tag

- rs2_tag

- rd_tag

- is_imm  (only for ALU and LSU)

- imm (only for ALU and LSU)

- br_type (only for ALU)

- is_jump

b) Modules

Rename Table

| archReg | specTag | comTag |
| --- | --- | --- |

- Shows latest mapping of architectural registers to physical registers (tags)

- specTag is the last physical register assigned to a given architectural register

- comTag is the last physical register committed for a given architectural register

- Tags are never assigned, unless they are free => Neither a specTag or a comTag

Example:

- x3’s comTag was p5 and specTag was p6

- After commit p6 became comTag of x3. Hence, p6 will now act as x3

- p5 is now free for assignment as a specTag

Tag Buffer

| tag | free | freeCom | ready |
| --- | --- | --- | --- |

- free field is marked 0, when a tag is assigned as specTag in rename table

- freeCom is marked 0, when a tag is committed, and assigned as a comTag in rename table

- Both free and freeCom are set, when a tag is freed

- tags are only freed, when replaced by a new comTag

- ready implies that result in a tag is ready to be read

- ready is marked 0, when a tag is assigned as specTag for an instruction

Issue Buffer (Arithmetic)

| sqN | pc | valid | oper | rs1_tag | ready1 | rs2_tag | ready2 | rd_tag | is_imm | imm | br_type | is_jump |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Holds an instruction’s

- Sequence number

- The operation for ALU

- Type of possible instruction. Based on the type of the instruction, decisions for oper 1 and 2 are taken. For instance, if it is a jump type. oper1 = pc and oper2 = 4, so ready1 and ready2 will be set automatically

- Tags of operands

- ready1 and ready2, if the corresponding operand is available

- Destination tag

- Valid tells if entry is valid

Purpose:

- Holds instructions until both operands valid (result becomes available)

- When both valids are set, instruction is issued without consideration of program order

- Allows OoO issue

- In case of stall, previous stages can continue to work unless the buffer is full

Reorder Buffer (ROB)

| sqN | pc | arch_tag | rd_tag | ready | valid |
| --- | --- | --- | --- | --- | --- |

When an instruction enters ROB, its

- Sequence number (SqN)

- Architectural Destination Register (arch_tag)

- Physical Destination Register (rd_tag)

- pc of that instruction, which can be used to restore program order in case of a flush are stored

- ready bit signifies that instruction is ready for commit

- valid bit signifies if entry is valid

Working:

- ready bit for an instruction is set when result is broadcasted and its rd_tag matches with broadcasted tag

- The two instructions at the head of ROB are committed in each cycle

- After commit the comTag mapping changes

- ROB allows instructions to be committed in order

Register File:

- Single register file in the core that only contains physical registers

- Architectural registers are just a concept. They don’t exist physically

- Result is written to this RF once computed

- Commit in this case implies a particular register becoming comTag for an architectural register

- For instance, if p5 is the comTag of x3, then p5 can be considered an architectural register

Instruction Flow

- A valid pc is passed to instruction aligner

  - It checks if the pc is 4 or 8 bit aligned

  - If 4 bit aligned, out of the 2 instructions from instruction memory, only the second instruction is passed as valid and consumed = 1

  - If 8 bit aligned, both instructions from instruction memory are passed as valid, and consumed = 2

  - Based on whether 1 or 2 instructions are passed as valid, the pc jumps by 4 or 8, respectively

  - First 8 bits of pc are set as 0, and passed as fetch_base to instruction memory, to keep the address 8 bit aligned

  - Then, the instruction aligner forwards 1 or 2 valid instructions to the fetch buffer

- Fetch buffer forwards at most two instructions from its head to decode

- In decode

  - If instruction is branch or jumpy type, the simple predictor always generates a jump signal

  - Based on the pc and immediate of that instruction, a target address is calculated for the jump

  - In case, if both instructions are jump/branch, the first one is given priority

  - This target address and jump signal, are forwarded to pc, bypassing pipeline register

  - jump signal also acts as flush for fetch buffer, clearing any pre-loaded instructions

- Decode stage register forwards at most two instructions to rename table

- In rename table

  - For the two instructions received, their rs (read) registers are checked against the archReg, in rename table → Corresponding specTags become rs_tags for that instruction

  - Tag buffer sends two free tags to rename table, which assigns those tags to the rd registers as rd_tags. Correspondingly, those tags are stored as specTags for that rd (archReg)

  - When a tag is set as specTag in rename table, its free is marked 0 in the tag buffer

  - The renamed instructions are forwarded to dispatch unit

  - For each valid instruction, we forward its sqN, pc, rd (arch_reg) and rd_tag to ROB, so it is stored there in program order

  - If the renamed instruction is a branch type, in that case, a snapshot of rename table and tag buffer is forwarded and stored in a branch checkpoint unit, along with that instructions sqN

  - At anytime, there can be no more than 8 branch instructions in the proceeding pipeline

  - If a 9th branch instruction comes to rename table, then the preceding pipeline is to be stalled